| ABC Petrochemicals Pvt. Ltd. | Document: INC-001 |
| --- | --- |
| Gujarat Manufacturing Plant | Rev: 1.0 ¦ Date: 15-Jan-2026 |

# Pump Oil Leakage Incident  --  P-101

Equipment: P-101  --  Cooling Water Circulation Pump

Incident Date: 03-Apr-2026 | Time: 14:30 hrs

Reported By: Vikram Singh (Shift Lead)

## Incident Description

During routine round at 14:30 hrs, operator observed a steady oil leak from the pump P-101 bearing housing. Approximately 2 litres of lubricating oil had pooled on the drip tray. The pump was operating normally with no abnormal vibration or noise.

## Immediate Actions Taken

Pump was taken offline and spare pump (P-101-S) was started

Oil spill contained using absorbent pads  --  area cleaned

Bearing housing drain plug found to be loose  --  tightened to 35 Nm

Fresh oil (Mobil Polyrex EM) replenished to correct level

Pump restarted and monitored for 1 hour  --  no further leakage

## Root Cause

The bearing housing drain plug was not fully tightened during the previous bearing replacement (MNT-002, 12-Mar-2026). Vibration during operation caused gradual loosening over 22 days of service.

## Corrective Actions

1. Added drain plug tightening to bearing replacement checklist.

2. Lock wire applied to drain plug to prevent back-off.

3. Maintenance engineer (Suresh Kumar) counselled on torque verification.

## Environmental Impact

Approximately 2 litres of oil contained within drip tray. No soil or water contamination. Absorbent pads disposed as hazardous waste.

Reported By: Vikram Singh  |  Reviewed By: Rajesh Mehta