| ABC Petrochemicals Pvt. Ltd. | Document: INS-002 |
| --- | --- |
| Gujarat Manufacturing Plant | Rev: 1.0 ¦ Date: 15-Jan-2026 |

# Vibration Analysis Report  --  C-201

Equipment: C-201  --  Plant Instrument Air Compressor

Survey Date: 10-May-2026

Analyst: Suresh Kumar (Maintenance Engineer)

## Methodology

Vibration data collected using SKF Microlog CMXA-75 analyzer. Measurements taken at 8 bearing points in X, Y, Z axes. Data compared against ISO 10816-3 criteria.

## Measurement Summary

| Point | X (mm/s) | Y (mm/s) | Z (mm/s) |
| --- | --- | --- | --- |
| Motor DE | 1.8 | 2.1 | 1.5 |
| Motor NDE | 2.0 | 2.3 | 1.7 |
| Comp DE | 3.5 | 4.1 | 2.9 |
| Comp NDE | 3.2 | 3.8 | 2.6 |

## Analysis

Overall vibration levels at compressor bearings are in the ALERT zone per ISO 10816-3 (2.8 - 4.5 mm/s, Class II). The dominant frequency peak at 1x RPM (2950 CPM) suggests slight unbalance. No bearing defect frequencies detected.

## Recommendations

1. Schedule dynamic balancing of compressor rotor during next planned shutdown.

2. Increase monitoring frequency to monthly until resolved.

3. Check foundation bolt torque during next weekly inspection.