| ABC Petrochemicals Pvt. Ltd. | Document: MNT-002 |
| --- | --- |
| Gujarat Manufacturing Plant | Rev: 1.0 ¦ Date: 15-Jan-2026 |

# Bearing Replacement Report  --  P-101

Equipment: P-101  --  Cooling Water Circulation Pump

Date: 12-Mar-2026

Work Order: WO-2026-0341

## Reason for Replacement

High vibration (7.8 mm/s) and elevated bearing temperature (78 deg C) observed during routine monitoring. Investigation indicated bearing wear on the drive-end (DE) bearing.

## Work Performed

Isolated P-101 and applied LOTO

Removed coupling guard and coupling halves

Extracted DE bearing (SKF 6319 C3) and NDE bearing (SKF 6320 C3)

Inspected bearing housing and shaft  --  no scoring

Installed new DE and NDE bearings using induction heater (max 110 deg C)

Reassembled coupling and aligned within 0.05 mm

Filled bearing housing with fresh grease (Mobil Polyrex EM)

Reinstalled coupling guard and removed LOTO

Performed test run for 2 hours

## Post-Repair Readings

Vibration (DE): 1.8 mm/s

Vibration (NDE): 2.1 mm/s

Bearing Temperature: 52 deg C (stabilized)

## Parts Used

SKF 6319 C3 (Qty 1)  --  Bearing No. B-6319-01

SKF 6320 C3 (Qty 1)  --  Bearing No. B-6320-01

Mobil Polyrex EM  --  Grease (500 g)

## Sign-Off

Performed By: Suresh Kumar  |  Verified By: Priya Sharma