| ABC Petrochemicals Pvt. Ltd. | Document: MNT-003 |
| --- | --- |
| Gujarat Manufacturing Plant | Rev: 1.0 ¦ Date: 15-Jan-2026 |

# Mechanical Seal Leakage Repair  --  P-102

Equipment: P-102  --  Condensate Return Pump

Date: 22-Feb-2026

Work Order: WO-2026-0287

## Problem Description

Operator reported visible drip leakage (~15 drops/min) from the mechanical seal area of pump P-102. Seal water flush line was at normal pressure.

## Root Cause

Mechanical seal faces worn beyond tolerance. Seal face exhibited circumferential scoring and one visible chip on the stationary face.

## Repair Actions

Isolated pump and performed LOTO

Drained pump casing and removed backplate

Extracted defective mechanical seal (John Crane 5610, 45mm)

Inspected shaft sleeve  --  minor wear marks, polished with fine emery

Installed new mechanical seal (John Crane 5610, 45mm) with clean assembly gel

Reassembled backplate and filled casing

Slowly pressurized and checked for leaks  --  zero leakage at 4.5 barg

Started pump and monitored for 1 hour  --  seal weepage < 3 drops/min

## Parts Used

John Crane 5610 Mechanical Seal, 45mm (Qty 1)  --  Part No. JC-5610-45

Gasket set (pump backplate)  --  P-102-GSKT-01

## Recommendation

Monitor seal water supply quality. Install Y-strainer on seal water line to prevent particulate damage to seal faces.

## Sign-Off

Performed By: Suresh Kumar  |  Approved By: Priya Sharma