| ABC Petrochemicals Pvt. Ltd. | Document: MAN-001 |
| --- | --- |
| Gujarat Manufacturing Plant | Rev: 1.0 ¦ Date: 15-Jan-2026 |

# Centrifugal Pump OEM Manual  --  P-101 / P-102

Equipment: P-101, P-102  --  Centrifugal Pumps

Supplier: Kirloskar Brothers Ltd.

Compiled By: Priya Sharma  |  Date: 20-Dec-2025

## 1. General Description

The KBL Series-C horizontal centrifugal pump is designed for continuous industrial service handling clean or slightly contaminated liquids. Models P-101 and P-102 are identical frame sizes with different impeller trims.

## 2. Technical Specifications

| Parameter | P-101 | P-102 |
| --- | --- | --- |
| Flow Rate | 180 m^3/hr | 120 m^3/hr |
| Head | 65 m | 50 m |
| Speed | 2950 RPM | 2950 RPM |
| Motor Power | 45 kW | 30 kW |
| Impeller Dia. | 240 mm | 210 mm |
| Material | CI / SS316 | CI / SS304 |
| Connection | 6" Flanged | 4" Flanged |

## 3. Installation

Foundation must be level within 0.05 mm/m. Grout after alignment. Suction piping must be straight for minimum 5 pipe diameters before pump suction. Use pipe supports to avoid nozzle loading.

## 4. Maintenance Schedule

• Daily: Check bearing temperature, seal leakage, vibration

• Monthly: Grease bearings, check coupling alignment

• Quarterly: Check impeller clearance, replace if worn

• Annually: Major overhaul  --  replace bearings, seals, gaskets

## 5. Troubleshooting

Low Flow: Check suction strainer, impeller clearance, NPSH available.

High Vibration: Check alignment, bearing condition, cavitation.

Seal Leakage: Replace mechanical seal; verify seal water quality.