| ABC Petrochemicals Pvt. Ltd. | Document: SOP-001 |
| --- | --- |
| Gujarat Manufacturing Plant | Rev: 1.0 ¦ Date: 15-Jan-2026 |

# Centrifugal Pump Start-Up Procedure

Equipment: P-101  --  Cooling Water Circulation Pump (Centrifugal Pump)

Department: Operations

Prepared By: Priya Sharma  |  Approved By: Rajesh Mehta

## 1. Purpose

To define the safe and systematic procedure for starting centrifugal pump P-101 to ensure reliable operation and prevent equipment damage.

## 2. Prerequisites

Valid work permit obtained

Suction line fully open and vented

Electrical supply verified (415V, 3-phase)

Seal cooling water supply ON

Coupling guard securely fitted

## 3. Start-Up Procedure

1. Verify lubrication oil level in bearing housing at sight-glass centerline.

2. Open suction valve fully; confirm positive suction pressure.

3. Open casing vent valve to release trapped air; then close.

4. Start drive motor from DCS or local push-button station.

5. Monitor discharge pressure rise to 4.5 barg within 5 seconds.

6. Slowly open discharge valve to 25%, then gradually to 100%.

7. Check motor current draw  --  must not exceed nameplate FLA (45 A).

8. Inspect mechanical seal  --  minor weepage acceptable during settling.

9. Verify pump vibration < 4.5 mm/s (alert limit).

10. Log all start-up parameters in shift log.

## 4. Safety Precautions

Ensure pump area is clear before starting.

Never start pump with suction valve closed.

Verify coupling guard is in place before energizing.

If abnormal vibration or noise occurs, stop immediately.

## 5. Revision History

| Rev | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 15-Jan-2026 | Initial Release | Priya Sharma |